# Requirement

- We wants to display boot-animation from board boot-up < 1.5 seconds.

  - Boot-animation’s length is ~2 seconds.

- At the end of boot-animation finish, SanSan’s HMI must display.

# Issue Description

- We are facing following issue:

“Reduce blank gap between boot animation end & SanSan HMI display (takes approx  5-6 second) “.

# Board Setup

- SanSan’s Y101-DEMO - 2017.05.22 Board

- Release 1.0.0 QNX binaries.

- Base line QNX BSP:

  - http://community.qnx.com/sf/frs/do/listReleases/projects.bsp/frs.freescale_i_mx6q_sabre_board_for?_pagesize=25

# Test-Steps

- Perform following steps to reproduce issue.

  - Power On

  - SanSan’s boot animation start displaying.

  - Once boot animation end, black/blank screen appears

  - Black/blank screen last long approx. 5-6 second.

  - SanSan’s HMI start displaying

# Current Execution flow

- POR the board, u-boot start executing

- U-boot load QNX images & execute QNX image

- QNX core kernel initialize core, configure pin control and start proconto service.

- Core kernel will give control to script for further execution.

- Script will perform following task:

  - Start common servers (e.g. sloggers, pipe)

  - Start the screen graphics

    - Install display driver (wfd cfg, screen)

      - This is without GPU libraries.

    - After finishes display driver initialization; we perform following process

    - Start running/displaying boot animation.

  - Let the other driver initialize.

  - Once, boot animation finishes (~2 seconds), we are executing following operation

    - Un-install display driver (wfd cfg, screen)

$ slay screen

    - Re-install display driver (wfd cfg, screen) & GPU libraries

$ screen

    - Start executing SanSan’s HMI application.

    - Which start displaying after ~5-6 seconds.

# Known facts of WFD driver

- WFD libraries, drivers & screen parameters can be configured by platform specific 'graphics.conf'

  - can be found @ <path to SDP 6.6>\target\qnx6\armle-v7\usr\lib\graphics\iMX6X\graphics.conf)

- Screen, WFD, GPU drivers are in library form

- We can just perform configuration using 'graphics.conf' file

- If all the screen, WFD, GPU drivers & libraries at its default locations (GRAPHICS_ROOT=/usr/lib/graphics/iMX6X):

  - install display driver (wfd cfg, screen) & GPU libraries

$ screen

  - This execution takes ~3-4 seconds to finish.

# Modification to display boot-animation < 1.5 seconds.

- To fulfill this requirement, we found that GPU libraries initialization takes much time

- Hence, we have placed Screen graphics libraries from its default locations (GRAPHICS_ROOT=/usr/lib/graphics/iMX6X) to secondary file partition.

  - Following GPU libraries are place at secondary file storage device.

    - libGalcore-3dvg.so

    - libGalcore-gc355.so

    - libEGL.so.1

    - libGLESv1_CM.so.1

    - libGLESv2.so.1

    - libEGL_viv.so

    - libGLES_CM_viv.so

    - libGLESv2_viv.so

    - libGLSLC.so

    - libOpenVG_viv.so

- From script (during QNX boot), we perform

  - install display driver (wfd cfg, screen)

$ screen

  - This execution takes ~100 milliseconds to finish.

- Start running/displaying boot animation.

  - The animation display start within ~200 milliseconds.

# Experiment to resolve issue

- Run display driver and boot animation in back ground

  - Install display driver (wfd cfg, screen) & GPU libraries in background

  - Execute boot animation in parallel (in background)

    - Boot animation can’t start display because

      - WFD driver takes ~3-4 seconds to install ‘screen’ device.